GraceEMO

Full Autonomous AI Humanoid Robot

AI/ML/DL  ·  ROS 2  ·  GPU Computing  ·  Real-Time MCU  ·  Autonomous Robotics

Project Proposal & Implementation Plan

Project Mentor: Dr. Mohit Arora
Lovely Professional University
School of Computer Science & Engineering
August 2026

# 1. Project Name

GraceEMO — Full Autonomous AI Humanoid Robot

# 2. Project Description

GraceEMO is an AI-native autonomous humanoid robot that can see, hear, understand, remember, learn, navigate, communicate, make decisions, and act autonomously in the real world. Originally built as a voice-interactive robot with animated facial expressions at Grace College of Engineering, the project is now being upgraded into a full autonomous robotic platform powered by GPU computing, ROS 2, and advanced AI/ML algorithms.

The robot integrates multiple AI/ML disciplines into a single cohesive system — combining natural language processing, computer vision, reinforcement learning, simultaneous localization and mapping (SLAM), and autonomous decision-making — to operate independently in real-world environments such as university campuses, labs, and event venues.

## How AI/ML is Used

Natural Language Processing (NLP) — Real-time speech-to-text (Vosk/Whisper), text-to-speech (Edge-TTS), and conversational AI via Large Language Models (Groq/OpenAI API) for human-like dialogue.

Computer Vision (CV) — YOLOv8 object detection with TensorRT GPU acceleration for real-time perception at 30+ FPS. MediaPipe for hand gesture and body pose recognition.

Face Recognition & Re-Identification — ArcFace/InsightFace deep learning models generate 512-dimensional face embeddings for identifying and remembering people across sessions.

SLAM & Navigation — GPU-accelerated Visual SLAM (Isaac ROS) and Nav2 path planning for autonomous mapping, localization, and obstacle avoidance using deep learning-based costmap generation.

Reinforcement Learning — Train navigation and interaction policies in NVIDIA Isaac Sim, then transfer learned behaviors to the physical robot (sim-to-real).

Multimodal Fusion — Deep learning-based fusion of vision, audio, sensor, and language modalities into a unified world model for situational awareness.

Memory-Augmented AI — Episodic, semantic, and spatial memory systems that augment LLM prompts with contextual recall — enabling the robot to remember past conversations, people, and places.

Behavior Trees + LLM Reasoning — LLM-powered goal decomposition combined with behavior trees for complex autonomous decision-making (e.g., "Find Sam" → navigate → search → approach → greet).

# 3. Modules

Module 1: Voice Interaction — Real-time speech recognition (STT), conversational AI (LLM), text-to-speech (TTS), emotion expression, and animated face GUI (PyGame).

Module 2: Computer Vision — Camera capture, YOLOv8 object detection, ArcFace face recognition, person re-identification, ByteTrack multi-object tracking, MediaPipe gesture recognition, and scene understanding.

Module 3: Hardware Control — ESP32 micro-ROS firmware for servo motors (head pan/tilt), DC motor differential drive, IMU orientation, ultrasonic/IR sensors, NeoPixel LED mood lighting, and emergency stop.

Module 4: Navigation & SLAM — GPU-accelerated SLAM, Nav2 autonomous path planning, person following, waypoint patrol, frontier exploration, and obstacle avoidance.

Module 5: Memory System — Persistent episodic memory (events), semantic memory (facts/knowledge), spatial memory (named map locations), person database (face → name → history), and memory-augmented LLM prompting.

Module 6: World Model — Multimodal sensor fusion, dynamic scene graph, situational context engine, and predictive modeling.

Module 7: AI Brain — Central autonomous controller with behavior trees, hierarchical state machine, LLM-based goal planner, proactive behavior engine, and safety monitor.

Module 8: Learning Pipeline — Data collection, model fine-tuning, reinforcement learning in simulation, imitation learning, and continuous deployment pipeline.

# 4. Tech Stack

| Category | Technology | Purpose |
| --- | --- | --- |
| Robotics Framework | ROS 2 Humble | Middleware, topic-based communication, launch system |
| GPU Computing | NVIDIA CUDA + TensorRT | Accelerated AI inference on Jetson |
| AI Framework | PyTorch + Ultralytics | YOLOv8 training, model development |
| Vision | OpenCV + MediaPipe + Isaac ROS | Image processing, gesture recognition, camera pipeline |
| Face Recognition | ArcFace / InsightFace | Face embedding and identification |
| NLP / LLM | Groq API (Compound-Mini) | Conversational AI with <150ms response |
| Speech-to-Text | Vosk (offline) / Whisper | Real-time voice recognition |
| Text-to-Speech | Edge-TTS | Natural voice synthesis |
| Navigation | Nav2 + SLAM Toolbox | Autonomous path planning and mapping |
| Simulation | NVIDIA Isaac Sim / Gazebo | Virtual testing before physical deployment |
| Real-Time MCU | micro-ROS (Arduino) | ESP32 ↔ ROS 2 bridge |
| MCU Firmware | PlatformIO + Arduino | ESP32 servo/motor/sensor firmware |
| GUI | PyGame | Animated robot face/eyes display |
| Database | SQLite + JSON | Persistent memory storage |
| Language | Python 3.11 + C++ (firmware) | Primary development languages |
| Build System | colcon + CMake | ROS 2 workspace build |
| Containerization | Docker | Reproducible dev environments |
| Version Control | Git + GitHub | Source code management |

# 5. Project Requirements: Checklist

## Hardware Requirements

☑  NVIDIA Jetson Orin Nano Super (8GB) — Main compute board

☑  ESP32-WROOM-32 — Real-time MCU for motor/sensor control

☐  USB Webcam or Pi Camera Module 3 — Vision input

☐  MG996R Servo Motors × 2 — Head pan/tilt

☐  PCA9685 16-Channel PWM Driver — Servo driver board

☐  TB6612FNG Motor Driver — Differential drive

☐  TT Gear Motors × 2 + Wheels — Locomotion

☐  MPU6050 IMU — Orientation sensing

☐  HC-SR04 Ultrasonic Sensors × 3 — Obstacle detection

☐  TCRT5000 IR Sensors × 2 — Cliff/edge detection

☐  WS2812B NeoPixel LED Strip — Mood lighting

☑  7-inch HDMI Display — Face/eyes output

☑  USB Microphone — Voice input

☑  Speaker (3W amplified) — Voice output

☐  5V 4A Power Supply + 7.4V LiPo Battery — Power

☐  RPLiDAR A1 (optional) — 360° 2D SLAM

☐  Intel RealSense D435i (optional) — Depth camera for Visual SLAM

## Software Requirements

☑  Ubuntu 22.04 LTS (Jetson) or Docker (Mac dev)

☐  ROS 2 Humble Hawksbill

☐  NVIDIA JetPack 6.x SDK

☐  NVIDIA Isaac ROS packages

☑  Python 3.11 + pip/venv

☐  PyTorch + Ultralytics (YOLOv8)

☐  OpenCV 4.x + MediaPipe

☐  PlatformIO (ESP32 firmware)

☐  micro-ROS Arduino library

☐  Docker Desktop (Mac development)

☑  Git + GitHub repository

☑  Vosk speech model (offline STT)

☑  Edge-TTS (text-to-speech)

☑  Groq API key (LLM inference)

# 6. Hardware Architecture

GraceEMO uses a two-tier compute architecture separating high-level AI processing from real-time hardware control:

## Tier 1: NVIDIA Jetson Orin Nano Super (High-Level Brain)

67 TOPS AI compute (1024-core Ampere GPU)

8 GB LPDDR5 unified memory

Runs ROS 2 Humble + Isaac ROS + Nav2

GPU-accelerated YOLOv8 inference (30+ FPS via TensorRT)

Visual SLAM, path planning, face recognition, LLM reasoning

Connects to camera, microphone, speaker, display via USB/HDMI/CSI

## Tier 2: ESP32-WROOM-32 (Real-Time MCU)

Dual-core 240 MHz Xtensa processor

Runs micro-ROS client firmware

Controls PCA9685 servo driver (head pan/tilt via I2C)

Controls TB6612FNG motor driver (differential wheel drive via PWM)

Reads MPU6050 IMU (orientation via I2C)

Reads HC-SR04 ultrasonic sensors (obstacle distance via GPIO)

Reads TCRT5000 IR sensors (cliff detection via analog)

Drives WS2812B NeoPixel LEDs (mood colors via data pin)

Hardware emergency stop and software watchdog timer

Connected to Jetson via USB Serial (micro-ROS Agent bridge)

## Bill of Materials

| Component | Model | Purpose | Est. Cost (₹) |
| --- | --- | --- | --- |
| Compute Board | Jetson Orin Nano Super 8GB | AI Brain (67 TOPS GPU) | 20,000 |
| Real-Time MCU | ESP32-WROOM-32 | Motor/servo/sensor control | 500 |
| Camera | USB Webcam / Pi Camera 3 | Computer vision | 1,500 |
| Servos × 2 | MG996R | Head pan/tilt | 400 |
| Servo Driver | PCA9685 | 16-ch PWM driver | 250 |
| Motor Driver | TB6612FNG | Dual H-bridge | 200 |
| DC Motors × 2 | TT Gear Motor + wheels | Differential drive | 200 |
| IMU | MPU6050 | Orientation sensing | 150 |
| Ultrasonic × 3 | HC-SR04 | Obstacle detection | 150 |
| IR Sensors × 2 | TCRT5000 | Cliff detection | 100 |
| LED Strip | WS2812B NeoPixel (30 LEDs) | Mood lighting | 300 |
| Display | 7" HDMI | Face/eyes (existing) | — |
| Speaker | 3W amplified | Voice output (existing) | — |
| Microphone | USB microphone | Voice input (existing) | — |
| Power Supply | 5V 4A + 7.4V LiPo | Logic + motor power | 1,500 |
| Miscellaneous | Wires, chassis, connectors | Assembly | 2,000 |
| TOTAL |  |  | ~27,250 |

# 7. Software Architecture

The software is organized as a modular ROS 2 workspace (graceemo_ws) with 10 independent packages, each responsible for a specific capability. All packages communicate via standardized ROS 2 topics, services, and actions.

| ROS 2 Package | Description |
| --- | --- |
| graceemo_msgs | Custom ROS 2 message, service, and action definitions |
| graceemo_description | URDF/Xacro robot model for tf2 transforms and simulation |
| graceemo_bringup | Launch files and system configuration (YAML parameters) |
| graceemo_voice | Voice interaction: STT → LLM → TTS + animated face GUI |
| graceemo_vision | Camera + YOLOv8 + face recognition + gesture + tracking |
| graceemo_hardware | ESP32 firmware + micro-ROS bridge + servo/motor management |
| graceemo_navigation | SLAM + Nav2 + person following + patrol + exploration |
| graceemo_memory | Episodic, semantic, spatial, procedural memory + person DB |
| graceemo_worldmodel | Multimodal fusion, scene graph, context, prediction |
| graceemo_brain | Behavior tree + state machine + LLM goal planner + safety |
| graceemo_learning | Data collection, model training, RL, imitation learning |

# 8. Expected Outcomes

1. A fully functional autonomous humanoid robot capable of independent operation in real-world environments (campus, lab, events).

2. Real-time natural voice conversation with context-aware, memory-augmented responses (<500ms latency).

3. GPU-accelerated computer vision: object detection (30+ FPS), face recognition (>95% accuracy), gesture recognition.

4. Autonomous navigation: SLAM-based mapping, path planning, obstacle avoidance, and person following.

5. Persistent memory: the robot remembers people, conversations, places, and events across sessions.

6. Multimodal AI: fuses vision, audio, sensors, and language into unified situational awareness.

7. Autonomous decision-making via behavior trees and LLM-based goal planning.

8. A modular, extensible ROS 2-based platform that serves as a teaching and experimentation tool for AI/Robotics labs.

9. Publishable research contributions in autonomous robotics, multimodal AI, and human-robot interaction.

10. Open-source codebase and documentation enabling replication at other institutions.

# 9. Team Composition

Project Mentor: Dr. Mohit Arora

Original Faculty Guide: Mrs. N. Nancy Chitra Thilaga, M.E., (Ph.D) — HoD, AI & Data Science, Grace College of Engineering

| Name | Role | Expertise | Responsibilities |
| --- | --- | --- | --- |
| Sam Davi | Project Lead | Sensors, Hardware, ROS 2 | System architecture, hardware integration, sensor fusion, ESP32 firmware, ROS 2 orchestration |
| John Allson | Hardware Lead | Mechanical, Electrical, Ideas | Robot chassis, mechanical design, electrical wiring, power systems, new feature ideation |
| Sarvesh | Software Lead | Software Development | ROS 2 node development, AI/ML pipelines, computer vision, voice interaction software |
| Alex Matthew | Design & Software | Software, UI/UX Design | Face GUI design, Blender animations, frontend interfaces, software development |

# 10. Prior Experience

The team has already built and demonstrated GraceEMO v1.0 — a working voice-interactive robot with the following proven capabilities:

Real-time voice conversation using Vosk STT + Groq LLM + Edge-TTS (< 500ms response latency)

Animated facial expressions (6 emotions: happy, sad, angry, focus, idle, intent) rendered via PyGame with Blender-animated eye videos

Knowledge base system: 8 text files covering persona, college info, courses, creators, facilities, and interaction protocols

Voice Activity Detection (VAD) with tuned energy thresholds for robust microphone input

Multi-threaded architecture: simultaneous voice recording, LLM inference, TTS playback, and GUI rendering

Built and demonstrated on Raspberry Pi 5 at Grace College of Engineering campus events

Successfully adapted from Raspberry Pi to macOS development environment with zero-lag audio pipeline

# 11. LPU Use Case

GraceEMO serves as both a functional autonomous robot and a modular teaching/experimentation platform for Lovely Professional University's Robotics and AI Lab.

## Campus Deployment

Welcome and guide visitors at campus entrance, events, and open days

Autonomous campus patrol — recognize and greet students/faculty by name

Interactive Q&A kiosk — answer questions about departments, facilities, admissions

Event assistant — navigate event venues, interact with attendees, provide information

## Robotics & AI Lab — Teaching Platform

Once the prototype is ready, GraceEMO becomes a hands-on teaching tool where students can experiment with different AI algorithms and observe real-time responses on a physical robot. This is the most important part of the Robotics and AI Lab.

Computer Vision Experiments — Students can swap YOLOv8 with SSD, Faster R-CNN, or DETR and compare detection accuracy and speed on the same hardware. Face recognition models (ArcFace vs FaceNet vs DeepFace) can be benchmarked.

Navigation Algorithms — Compare DWB local planner vs TEB vs MPPI. Test different SLAM algorithms (GMapping vs Cartographer vs SLAM Toolbox). Students observe path planning differences in real-time.

NLP & Conversational AI — Swap LLM backends (GPT-4, Llama 3, Gemma, Mistral) and compare conversation quality, response time, and persona adherence. Experiment with prompt engineering techniques.

Reinforcement Learning — Train navigation policies in Isaac Sim with different RL algorithms (PPO, SAC, TD3) and deploy to the physical robot to observe sim-to-real transfer.

Sensor Fusion — Experiment with different sensor combinations (camera-only vs camera+LiDAR vs camera+ultrasonic) and compare navigation reliability.

Human-Robot Interaction (HRI) — Study user engagement metrics, conversation patterns, and emotional responses. Conduct A/B tests with different personality configurations.

Behavior Trees & Planning — Students design custom behavior trees for new autonomous behaviors and test them on the physical robot in minutes.

Edge AI & Model Optimization — Compare TensorRT vs ONNX Runtime vs raw PyTorch inference. Experiment with model quantization (FP16, INT8) and observe accuracy/speed tradeoffs.

# 12. Project Timelines

| Phase | Timeline | Milestones | Deliverables | Status |
| --- | --- | --- | --- | --- |
| Phase 0: Voice Robot v1.0 | Jan – Jul 2026 (Completed) | • Voice conversation working • 6 animated emotions • Knowledge base loaded • Campus demo completed | • Working voice-interactive robot • PyGame face GUI • Vosk + Groq + Edge-TTS pipeline | ✅ Done |
| Phase 1: ROS 2 Foundation | Aug – Sep 2026 (4 weeks) | • ROS 2 workspace builds • Voice node publishes topics • Launch file starts system • URDF model created | • graceemo_ws workspace • graceemo_msgs package • voice_node.py + face_gui_node.py • voice_only_launch.py | 🔜 Next |
| Phase 2: Computer Vision | Sep – Oct 2026 (4 weeks) | • YOLOv8 runs at 30+ FPS • Face DB stores/recalls people • Gesture recognition working • Vision-voice bridge active | • graceemo_vision package • TensorRT model engines • Face recognition database • Person tracker node | 📋 Planned |
| Phase 3: Hardware Control | Oct – Nov 2026 (4 weeks) | • ESP32 firmware flashed • Servos respond to ROS topics • Motors drive via /cmd_vel • All sensors publishing data | • graceemo_hardware package • ESP32 micro-ROS firmware • micro-ROS Agent bridge • Hardware safety system | 📋 Planned |
| Phase 4: Navigation & SLAM | Nov – Dec 2026 (4 weeks) | • Room map generated • Autonomous point-to-point nav • Person following works • Obstacle avoidance reliable | • graceemo_navigation package • Saved maps directory • Nav2 configuration tuned • Patrol waypoint system | 📋 Planned |
| Phase 5: Memory System | Dec 2026 – Jan 2027 (3 weeks) | • Person DB recalls names • Past conversations referenced • Named locations navigable • Memory-augmented LLM active | • graceemo_memory package • SQLite persistent database • Memory query service • Augmented prompt system | 📋 Planned |
| Phase 6: World Model | Jan – Feb 2027 (4 weeks) | • Multimodal fusion working • Scene graph generated • Context-aware responses • Prediction engine active | • graceemo_worldmodel package • Scene graph visualizer • Context engine • World state topic publishing | 📋 Planned |
| Phase 7: AI Brain | Feb – Mar 2027 (4 weeks) | • Fully autonomous operation • Behavior tree executing • Goal planning working • Safety monitor active | • graceemo_brain package • Behavior tree definitions • State machine implementation • Safety rules configuration | 📋 Planned |
| Phase 8: Learning Pipeline | Mar 2027 → (Ongoing) | • Data collection running • Models improving • RL policy trained in sim • Deployed to real robot | • graceemo_learning package • Training scripts • Evaluation metrics • Model versioning system | 📋 Planned |

# 13. Project Deployment

## Development Environment

macOS development using Docker containers with ROS 2 Humble + Isaac ROS dev images

colcon build + test on Mac → verify all nodes compile and pass unit tests

Gazebo / Isaac Sim simulation for navigation testing without physical hardware

Git + GitHub for version control with CI/CD integration

## Physical Deployment

Flash NVIDIA JetPack 6.x + Ubuntu 22.04 on Jetson Orin Nano Super

Install ROS 2 Humble + Isaac ROS packages natively on Jetson

Flash ESP32 firmware via PlatformIO over USB

Connect all hardware: camera, servos, motors, sensors, display, mic, speaker

Run: ros2 launch graceemo_bringup robot_launch.py — single command starts entire system

Deploy saved maps and face databases to Jetson persistent storage

## Deployment Targets

LPU Robotics & AI Lab — primary deployment for teaching and experimentation

LPU campus entrance / visitor center — welcome and guide visitors

Academic conferences and exhibitions — live demonstrations

Grace College of Engineering — original home campus deployment

# 14. Teaching & Experimentation Platform for Robotics and AI Lab

Once the prototype is operational, GraceEMO becomes a powerful teaching platform where students can experiment with different AI algorithms and observe real-time responses on a physical robot. This is the most important part of integrating GraceEMO into the Robotics and AI Lab.

## How It Works as a Teaching Tool

GraceEMO's modular ROS 2 architecture means every AI component is a swappable node. Students can replace any module with their own implementation and immediately see the effect on the physical robot — creating a unique learn-by-doing experience impossible with simulation alone.

Experiment 1: Object Detection Benchmarking — Swap the YOLOv8 detector node with SSD MobileNet, Faster R-CNN, EfficientDet, or DETR. Compare detection accuracy (mAP), inference speed (FPS), and GPU memory usage on the same Jetson hardware. Observe how each model handles real-world occlusion, lighting, and distance.

Experiment 2: Face Recognition Algorithm Comparison — Replace ArcFace with FaceNet, DeepFace, or custom-trained models. Measure recognition accuracy across lighting conditions, angles, and distances. Students learn embedding-based recognition vs. classification-based approaches.

Experiment 3: Navigation Policy Testing — Compare Nav2 planners (DWB vs TEB vs MPPI) and observe real-time path differences. Test SLAM algorithms (GMapping vs Cartographer vs SLAM Toolbox) and compare map quality. Students see theory become physical robot movement.

Experiment 4: Reinforcement Learning Sim-to-Real — Train navigation policies in Isaac Sim using PPO, SAC, or TD3. Deploy trained models to the physical robot and observe sim-to-real transfer gaps. Students learn domain adaptation firsthand.

Experiment 5: LLM Personality & Reasoning — Swap conversation backends (GPT-4, Llama 3, Gemma 2, Mistral) and compare response quality, persona adherence, reasoning depth, and latency. Experiment with chain-of-thought prompting, retrieval-augmented generation (RAG), and few-shot learning.

Experiment 6: Sensor Fusion Ablation Study — Systematically disable sensors (remove camera, remove ultrasonic, remove IMU) and measure navigation degradation. Students learn the importance of sensor redundancy and fusion.

Experiment 7: Edge AI Optimization — Compare TensorRT FP16 vs INT8 vs FP32 inference. Measure accuracy loss vs. speed gain. Experiment with model pruning, knowledge distillation, and ONNX export workflows.

Experiment 8: Human-Robot Interaction Studies — Conduct controlled experiments measuring user engagement, trust, and satisfaction with different personality configurations, voice tones, and proactive vs. reactive behaviors.